# General Lab Procedures

Data Acquisition and Calibration

- Review the accompanying presentation slides [https://docs.google.com/presentation/d/1B35zL_IkINuuKQ8NIXFKNZCMMRDfD0Q6fm9fiU3JGMw/edit?usp=sharing] on data collection and analysis, as the procedures are applicable to both the Lead-210 and Cosmic Ray segments of this lab.

- Calibration Procedure: The Tracker software provides particle track data in pixels. To convert pixel measurements to centimeters (cm), an object of known size within the Petri dish must be measured to establish the pixels-to-cm conversion factor.

- Data Archival: All raw video files and processed data spreadsheets must be uploaded to the shared drive location

Background Physics and Theory

- The provided presentation slides [https://docs.google.com/presentation/d/1Ke5z5PG0GqcO819rRQGf8dI0Vj1WmEbkjeAe4o_L0DA/edit?usp=sharing] cover the foundational physics and initial data analysis techniques for this experiment. Successful completion of this upper-level lab requires independent research beyond the provided materials.

# Laboratory Techniques extra material

Cosmic Ray Background Measurement

- Objective: Estimate the angular distribution of cosmic rays.

- Data Collection (Background): Acquire multiple video recordings of the cloud chamber in the absence of the radioactive source. The goal is to detect high-energy cosmic ray tracks, characterized by strong, definite ionization paths.

  - Operational Note: Cosmic ray tracks are typically visible every 30–60 seconds. Record sufficient video time to collect a minimum of 30 distinct tracks. Excessive alcohol condensation requires wiping down the lid and possibly reapplying alcohol to the chamber.

- Calibration Check: Confirm that the calibration required for converting pixel measurements to track length remains consistent.

- Track Recording: Record the timestamp for each of the collected cosmic ray tracks.

- Data Analysis & Plotting:

  - Generate a plot showing the trajectory of all observed tracks.

  - Plot the arrival time of each track versus its measured track length.

- Angular Distribution: Estimate the incoming angle of the cosmic ray based on the track length, as discussed in the lab session. Plot the resulting angular distribution. Note: Given the minimum target of 30 tracks, the binning may be coarse.

- Discussion Points:

  - Identify and discuss the factors that influence the accuracy of the estimated angular distribution.

  - Describe the expected shape of the angular distribution based on theoretical models and previous lab discussions.